# Traffic Simulation Documentation

# Intersection Data: Intersection

| Approach Name | Number of Lanes | Saturation Flow/Lane | Saturation Flow Total |
| --- | --- | --- | --- |
| 1_in | 2 | 1850.0 | 3700.0 |
| 1_out | 2 | 1850.0 | 3700.0 |
| 3_in | 2 | 1850.0 | 3700.0 |
| 3_out | 2 | 1850.0 | 3700.0 |
| 4_in | 2 | 1850.0 | 3700.0 |
| 4_out | 2 | 1850.0 | 3700.0 |

# Signal Plan

## Phase 1

Green: 68.0s | Amber: 3.0s | All Red: 3.0s | Start: 0.0s | Duration: 74.0s

| From Approach | Lane | To Approach | Movement Type | Average Flow (veh/h) | Saturation Flow/Lane (veh/h) |
| --- | --- | --- | --- | --- | --- |
| 1_in | 0 | 3_out | straight | 422.0 | 1850.0 |
| 1_in | 1 | 3_out | straight | 422.0 | 1850.0 |
| 1_in | 0 | 4_out | right | 101.0 | 1850.0 |
| 3_in | 0 | 1_out | straight | 414.5 | 1850.0 |
| 3_in | 1 | 1_out | straight | 414.5 | 1850.0 |

## Phase 2

Green: 32.0s | Amber: 3.0s | All Red: 3.0s | Start: 74.0s | Duration: 38.0s

| From Approach | Lane | To Approach | Movement Type | Average Flow (veh/h) | Saturation Flow/Lane (veh/h) |
| --- | --- | --- | --- | --- | --- |
| 4_in | 1 | 1_out | left | 94.0 | 1850.0 |
| 4_in | 0 | 3_out | right | 339.0 | 1850.0 |

## Phase 3

Green: 28.0s | Amber: 3.0s | All Red: 3.0s | Start: 112.0s | Duration: 34.0s

| From Approach | Lane | To Approach | Movement Type | Average Flow (veh/h) | Saturation Flow/Lane (veh/h) |
| --- | --- | --- | --- | --- | --- |
| 3_in | 0 | 1_out | straight | 414.5 | 1850.0 |
| 3_in | 1 | 1_out | straight | 414.5 | 1850.0 |
| 3_in | 1 | 4_out | left | 269.0 | 1850.0 |
| 4_in | 0 | 3_out | right | 339.0 | 1850.0 |

# Simulation Report

| Metric | Value |
| --- | --- |
| Vehicle Count | 2429 |
| Avg Delay Timeloss | 33.08 |
| Total Delay Timeloss | 80349.45 |
| Avg Waiting Time | 22.27 |
| Total Waiting Time | 54099.00 |
| Avg Stops | 0.59 |
| Total Stops | 1444.00 |
| Avg Queue Length | 2.67 |
| Total Queue Length | 57607.00 |
| Avg Duration | 69.51 |
| Total Duration | 168828.00 |
| Fitness Score | 21.54 |
| Weights | {'delay': 0.35, 'queue': 0.25, 'stops': 0.2, 'duration': 0.1, 'waiting': 0.1} |

# Signal Plan

## Phase 1

Green: 26.0s | Amber: 3.0s | All Red: 1.0s | Start: 0.0s | Duration: 30.0s

| From Approach | Lane | To Approach | Movement Type | Average Flow (veh/h) | Saturation Flow/Lane (veh/h) |
| --- | --- | --- | --- | --- | --- |
| 1_in | 0 | 3_out | straight | 422.0 | 1850.0 |
| 1_in | 1 | 3_out | straight | 422.0 | 1850.0 |
| 1_in | 0 | 4_out | right | 101.0 | 1850.0 |
| 3_in | 0 | 1_out | straight | 414.5 | 1850.0 |
| 3_in | 1 | 1_out | straight | 414.5 | 1850.0 |

## Phase 2

Green: 11.0s | Amber: 3.0s | All Red: 1.0s | Start: 30.0s | Duration: 15.0s

| From Approach | Lane | To Approach | Movement Type | Average Flow (veh/h) | Saturation Flow/Lane (veh/h) |
| --- | --- | --- | --- | --- | --- |
| 4_in | 1 | 1_out | left | 94.0 | 1850.0 |
| 4_in | 0 | 3_out | right | 339.0 | 1850.0 |

## Phase 3

Green: 15.0s | Amber: 3.0s | All Red: 1.0s | Start: 45.0s | Duration: 19.0s

| From Approach | Lane | To Approach | Movement Type | Average Flow (veh/h) | Saturation Flow/Lane (veh/h) |
| --- | --- | --- | --- | --- | --- |
| 3_in | 0 | 1_out | straight | 414.5 | 1850.0 |
| 3_in | 1 | 1_out | straight | 414.5 | 1850.0 |
| 3_in | 1 | 4_out | left | 269.0 | 1850.0 |
| 4_in | 0 | 3_out | right | 339.0 | 1850.0 |

# Simulation Report

| Metric | Value |
| --- | --- |
| Vehicle Count | 2439 |
| Avg Delay Timeloss | 18.55 |
| Total Delay Timeloss | 45238.11 |
| Avg Waiting Time | 8.87 |
| Total Waiting Time | 21629.00 |
| Avg Stops | 0.56 |
| Total Stops | 1369.00 |
| Avg Queue Length | 1.09 |
| Total Queue Length | 23460.00 |
| Avg Duration | 54.97 |
| Total Duration | 134066.00 |
| Fitness Score | 13.26 |
| Weights | {'delay': 0.35, 'queue': 0.25, 'stops': 0.2, 'duration': 0.1, 'waiting': 0.1} |